MECANUM NAV DRL

ARCHITECTURE SPECIFICATION

ROS 2 Jazzy + Gazebo Harmonic + SLAM + Nav2 + PPO cho robot Mecanum
ARCHITECTURE_FROZEN | IMPLEMENTATION_READY | NOT_YET_REAL_ROBOT_VALIDATED

| Thuộc tính | Giá trị |
| --- | --- |
| Trạng thái | ARCHITECTURE_FROZEN ¦ IMPLEMENTATION_READY ¦ NOT_YET_REAL_ROBOT_VALIDATED |
| Nền tảng | Ubuntu 24.04; ROS 2 Jazzy; Gazebo Harmonic |
| Robot | 4 bánh Mecanum; simulation RPLiDAR A1M8; robot thật YDLIDAR X3 |
| Điều hướng | SLAM Toolbox mapping; AMCL Omni; Nav2 global planning; Nav2 baseline; PPO hybrid local |
| Điều khiển thấp | Raspberry Pi 5 + STM32; wheel odometry -> EKF; hardware watchdog/E-stop |
| Đã khóa | Ownership, interfaces, TF, modes, observation/action, failure semantics, map identity, tests |
| Còn chờ | TBD_MEASURED và TBD_PROJECT_ACCEPTANCE; Gate 3-4 evidence |
| Không tuyên bố | Real-robot validation, Sim-to-Real success hoặc functional safety certification |
| Ngày phát hành | 16/09/2026 |

Cảnh báo: Architecture-Frozen nghĩa là có thể triển khai code theo contract này; deploy_real vẫn bị khóa cho tới khi hoàn tất đo đạc, HIL và phê duyệt Gate 4.

# Mục lục

| Mục 1-26 | Mục 27-51 và phụ lục |
| --- | --- |
| 1. Trạng thái và taxonomy | 27. Evaluation và thống kê |
| 2. Nguyên tắc phụ thuộc | 28. Acceptance preregistration |
| 3. Workspace và ownership | 29. Domain randomization |
| 4. Typed configuration | 30. Deployment robustness |
| 5. Profiles và modes | 31. Command arbitration |
| 6. Public protocols | 32. Safety runtime |
| 7. Transition dataclasses | 33. Holonomic collision shield |
| 8. SafetyStateSnapshot | 34. Stopping distance và E-stop |
| 9. ROS runtime | 35. ROS packaging |
| 10. Clock domains | 36. Startup validators |
| 11. Topic contract | 37. Test matrix |
| 12. QoS contract | 38. Gate 1-4 |
| 13. TF contract | 39. Measurement registry |
| 14. TF failure policy | 40. Traceability matrix |
| 15. Collision instrumentation | 41. State estimation và wheel odometry |
| 16. Contact latch | 42. Bốn chế độ vận hành |
| 17. Synchronization | 43. Mapping và localization |
| 18. Transactional reset | 44. HybridNavigationManager |
| 19. Gymnasium contract | 45. GlobalPath và LocalReference |
| 20. Observation contract | 46. Mission completion |
| 21. Action và command | 47. Command envelope và reset |
| 22. Reward contract | 48. Map artifact identity |
| 23. Termination | 49. Lifecycle startup order |
| 24. Task scenario seed | 50. Verification obligations |
| 25. PPO failure recovery | 51. Definition of Done |
| 26. Checkpoint và resume | Phụ lục A-D và kết luận |

# 1. Trạng thái và taxonomy đầu vào

| Token | Ý nghĩa | Có được còn trong resolved profile? |
| --- | --- | --- |
| FIXED_ARCH | Quyết định kiến trúc đã khóa | Có; thay đổi cần version/schema mới |
| SIM_BASELINE | Giá trị dùng riêng simulation | Có trong sim_*; cấm suy ra real |
| TBD_MEASURED | Phải đo robot thật và có registry VALID | Không trong deploy_real/Gate 4 |
| TBD_PROJECT_ACCEPTANCE | Chủ dự án phê duyệt trước final evaluation | Không trong preregistered evaluation |
| REQUIRED_BUILD_INPUT | Lấy từ metadata hoặc source robot | Không trong build/deploy resolved output |

Trạng thái phát hành là ARCHITECTURE_FROZEN, IMPLEMENTATION_READY và NOT_YET_REAL_ROBOT_VALIDATED. Các contract đã đủ rõ để triển khai; deploy_real vẫn bị khóa bởi TBD_MEASURED, HIL và Gate 4.

ConfigCompiler từ chối token chưa hợp lệ theo target; không thay placeholder bằng default im lặng.

Architecture-Frozen và Implementation-Ready không có nghĩa implementation đã vượt test, PPO đã đạt chất lượng, Sim-to-Real đã thành công hoặc hệ thống đạt chứng nhận functional safety.

## 1.1 Decision register đã khóa

| Decision ID | Quyết định cuối | Trạng thái |
| --- | --- | --- |
| D01 | Cây TF chính map->odom->base_link->lidar; GT chỉ là topic evaluator | FIXED_ARCH |
| D02 | mecanum_base_bridge tạo wheel odometry; chỉ EKF phát odom->base_link | FIXED_ARCH |
| D03 | RuntimeProfile và NavigationMode độc lập; compiler kiểm tra ma trận hợp lệ | FIXED_ARCH |
| D04 | PPO chỉ local controller trong hybrid; Nav2 Controller chỉ baseline | FIXED_ARCH |
| D05 | Observation 81-D dùng 72 angular sectors + LocalReference d/sin/cos | FIXED_ARCH |
| D06 | CommandEnvelope là ROS interface có epoch/generation/source instance | FIXED_ARCH |
| D07 | SafetySupervisor là tầng hard-limit và publisher duy nhất /cmd_vel | FIXED_ARCH |
| D08 | AMCL Omni hoặc SLAM localization sở hữu map->odom, không đồng thời | FIXED_ARCH |
| D09 | artifacts/maps là map source of truth với canonical manifest hash | FIXED_ARCH |
| D10 | Position-only mission goal bản đầu; settle kiểm tra vx,vy,wz liên tục | FIXED_ARCH |
| D11 | Physical E-stop độc lập Linux/ROS/DDS/Python | FIXED_ARCH |
| D12 | Gate 3-4 và real limits chỉ được kết luận từ evidence đo | FIXED_ARCH |
| D13 | sim_train dùng hidden GT -> SimulatedOdometryEmulator nội bộ -> noisy pose/twist snapshot; GT bị cấm khỏi policy; MVP không public odometry/TF; ACR S1 | FIXED_ARCH |

# 2. Nguyên tắc và dependency direction

mecanum_nav_bringup -> ConfigCompiler -> ResolvedConfig + AgentSpec
  -> EKF | SLAM/AMCL | Nav2 Planner | Nav2 baseline controller
  -> Trainer | Evaluator | HybridNavigationManager | DeploymentRuntime

wheel telemetry -> mecanum_base_bridge -> /wheel/odometry_raw -> EKF
map->odom -> odom->base_link -> base_link->lidar_frame
global path -> LocalReference -> PPO local controller
typed command sources -> CommandEnvelope -> SafetySupervisor -> /cmd_vel
physical E-stop -> motor enable/power path outside ROS/DDS/Python

Core không import rclpy, Gazebo hoặc SB3; runtime không tự đọc YAML.

Reward/termination/observation dùng cùng TransitionContext; safety dùng SafetyStateSnapshot độc lập.

Ground truth chỉ vào SimulatedOdometryEmulator nội bộ, TrainingTaskOracle, reset transaction/validation, reward, success/termination và evaluator theo profile. PolicyNode, ObservationEncoder, SnapshotSynchronizer và policy runtime không subscribe /ground_truth/odom, không nhận Gazebo world/model pose và không nhận raw GT buffer/object. ACR S1 Simulated Odometry And Ground Truth Isolation là quyết định áp dụng cho sim_train.

Một edge TF có đúng một authority; một actuator topic có đúng một publisher được phép.

# 3. Workspace và file ownership

~/mecanum_autonomy_ws/ros2_ws/
├── src/
│   ├── ROBOT_URDF_final_description/              // Package robot/Gazebo hiện có
│   │   ├── package.xml                             ✔
│   │   ├── CMakeLists.txt                          ✔
│   │   ├── launch/                                 ✔
│   │   ├── urdf/                                   ✔
│   │   ├── meshes/                                 ✔
│   │   ├── worlds/
│   │   ├── models/
│   │   ├── config/                                 ✔
│   │   └── rviz/
│   │
│   ├── mecanum_nav_rl_interfaces/                 // Typed ROS interfaces dùng chung
│   │   ├── package.xml                             ✔
│   │   ├── CMakeLists.txt                          ✔
│   │   ├── CHANGELOG.md
│   │   ├── msg/
│   │   │   ├── Heartbeat.msg                       ✔
│   │   │   ├── SafetyState.msg                     ✔
│   │   │   ├── NavigationState.msg
│   │   │   └── CommandEnvelope.msg
│   │   └── test/
│   │       └── test_interface_contracts.py
│   │
│   ├── mecanum_base_bridge/                       // Pi 5 <-> STM32 và wheel odometry
│   │   ├── package.xml
│   │   ├── setup.py
│   │   ├── setup.cfg
│   │   ├── resource/mecanum_base_bridge
│   │   ├── config/
│   │   │   ├── transport.yaml
│   │   │   ├── wheel_geometry.yaml
│   │   │   └── protocol.yaml
│   │   ├── mecanum_base_bridge/
│   │   │   ├── __init__.py
│   │   │   ├── bridge_node.py
│   │   │   ├── transport.py
│   │   │   ├── telemetry_decoder.py
│   │   │   ├── telemetry_state.py
│   │   │   ├── wheel_order.py
│   │   │   ├── wheel_odometry.py
│   │   │   ├── command_encoder.py
│   │   │   ├── heartbeat.py
│   │   │   └── watchdog.py
│   │   └── test/
│   │       ├── test_protocol_decode.py
│   │       ├── test_counter_wrap.py
│   │       ├── test_packet_loss.py
│   │       ├── test_mcu_reboot.py
│   │       └── test_wheel_odometry.py
│   │
│   ├── mecanum_nav_rl/                            // DRL, hybrid navigation và safety
│   │   ├── package.xml                             ✔
│   │   ├── setup.py                                ✔
│   │   ├── setup.cfg                               ✔
│   │   ├── README.md
│   │   ├── CHANGELOG.md                            ✔
│   │   ├── resource/mecanum_nav_rl                 ✔
│   │   ├── config/
│   │   │   ├── base.yaml                           ✔
│   │   │   ├── profiles/                           ✔
│   │   │   │   ├── sim_train.yaml                  ✔
│   │   │   │   ├── sim_eval.yaml
│   │   │   │   ├── deploy_sim.yaml
│   │   │   │   └── deploy_real.yaml
│   │   │   ├── algorithms/ppo.yaml
│   │   │   ├── qos.yaml
│   │   │   ├── topics.yaml
│   │   │   ├── frames.yaml
│   │   │   ├── acceptance.yaml
│   │   │   └── measurements.yaml
│   │   ├── scenarios/
│   │   │   ├── manifests/{train,validation,final_test}.yaml
│   │   │   ├── train/
│   │   │   ├── validation/
│   │   │   └── final_test/
│   │   ├── generated/
│   │   │   ├── collision_contract.yaml
│   │   │   ├── footprint.yaml
│   │   │   └── schema_hashes.yaml
│   │   ├── mecanum_nav_rl/
│   │   │   ├── __init__.py                         ✔
│   │   │   ├── core/
│   │   │   │   ├── __init__.py                     ✔
│   │   │   │   ├── types.py                        ✔
│   │   │   │   ├── frozen_array.py                 ✔
│   │   │   │   ├── snapshots.py                    ✔
│   │   │   │   ├── transition.py                   ✔
│   │   │   │   ├── protocols.py                    ✔
│   │   │   │   ├── enums.py                        ✔
│   │   │   │   └── exceptions.py                   ✔
│   │   │   ├── config/
│   │   │   │   ├── __init__.py                     ✔
│   │   │   │   ├── models.py                       ✔
│   │   │   │   ├── loader.py                       ✔
│   │   │   │   ├── compiler.py                     ✔
│   │   │   │   ├── hashing.py                      ✔
│   │   │   │   ├── agent_spec.py                   ✔
│   │   │   │   └── validators.py
│   │   │   ├── ros/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── runtime.py
│   │   │   │   ├── callback_groups.py
│   │   │   │   ├── clocks.py
│   │   │   │   ├── epoch_tracker.py
│   │   │   │   ├── sensor_buffers.py
│   │   │   │   ├── synchronizer.py
│   │   │   │   ├── qos_profiles.py
│   │   │   │   ├── tf_provider.py
│   │   │   │   ├── service_clients.py
│   │   │   │   └── endpoint_probe.py
│   │   │   ├── localization/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── models.py
│   │   │   │   ├── monitor.py
│   │   │   │   └── health.py
│   │   │   ├── navigation/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── modes.py
│   │   │   │   ├── models.py
│   │   │   │   ├── nav2_path_provider.py
│   │   │   │   ├── hybrid_navigation_manager.py
│   │   │   │   ├── lookahead.py
│   │   │   │   ├── goal_checker.py
│   │   │   │   ├── path_health.py
│   │   │   │   └── navigation_state_publisher.py
│   │   │   ├── simulation/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── adapter.py
│   │   │   │   ├── action_barrier.py
│   │   │   │   ├── reset_transaction.py
│   │   │   │   ├── contact_source.py
│   │   │   │   ├── contact_latch.py
│   │   │   │   ├── entity_control.py
│   │   │   │   └── collision_extractor.py
│   │   │   ├── env/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── mecanum_env.py
│   │   │   │   ├── episode_state.py
│   │   │   │   ├── info_builder.py
│   │   │   │   └── checker.py
│   │   │   ├── observations/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── encoder.py
│   │   │   │   ├── lidar.py
│   │   │   │   ├── goal_features.py
│   │   │   │   ├── motion.py
│   │   │   │   ├── space.py
│   │   │   │   └── validators.py
│   │   │   ├── actions/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── decoder.py
│   │   │   │   ├── limits.py
│   │   │   │   ├── space.py
│   │   │   │   └── command.py
│   │   │   ├── rewards/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── base.py
│   │   │   │   ├── manager.py
│   │   │   │   ├── navigation.py
│   │   │   │   ├── safety.py
│   │   │   │   ├── motion.py
│   │   │   │   └── validators.py
│   │   │   ├── termination/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── manager.py
│   │   │   │   ├── task_conditions.py
│   │   │   │   ├── time_limit.py
│   │   │   │   └── result.py
│   │   │   ├── tasks/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── task_state.py
│   │   │   │   ├── scenario.py
│   │   │   │   ├── manifest.py
│   │   │   │   ├── samplers.py
│   │   │   │   └── curriculum.py
│   │   │   ├── randomization/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── manager.py
│   │   │   │   ├── sensors.py
│   │   │   │   ├── timing.py
│   │   │   │   ├── dynamics.py
│   │   │   │   └── environment.py
│   │   │   ├── training/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── trainer.py
│   │   │   │   ├── env_factory.py
│   │   │   │   ├── model_factory.py
│   │   │   │   ├── callbacks.py
│   │   │   │   ├── checkpoint.py
│   │   │   │   ├── normalization.py
│   │   │   │   ├── seed_manager.py
│   │   │   │   └── run_supervisor.py
│   │   │   ├── evaluation/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── evaluator.py
│   │   │   │   ├── metrics.py
│   │   │   │   ├── reliability.py
│   │   │   │   ├── recorder.py
│   │   │   │   ├── statistics.py
│   │   │   │   ├── baseline_runner.py
│   │   │   │   └── report.py
│   │   │   ├── deployment/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── policy_node.py
│   │   │   │   ├── policy_loader.py
│   │   │   │   ├── inference.py
│   │   │   │   ├── command_adapters.py
│   │   │   │   └── real_robot_adapter.py
│   │   │   ├── safety/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── sensor_node.py
│   │   │   │   ├── supervisor.py
│   │   │   │   ├── health.py
│   │   │   │   ├── freshness.py
│   │   │   │   ├── command_validator.py
│   │   │   │   ├── command_arbiter.py
│   │   │   │   ├── limits.py
│   │   │   │   ├── footprint.py
│   │   │   │   ├── collision_shield.py
│   │   │   │   ├── stopping_distance.py
│   │   │   │   ├── watchdog.py
│   │   │   │   └── final_twist_publisher.py
│   │   │   ├── validation/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── config_validator.py
│   │   │   │   ├── graph_validator.py
│   │   │   │   ├── tf_validator.py
│   │   │   │   ├── qos_validator.py
│   │   │   │   ├── schema_validator.py
│   │   │   │   ├── map_validator.py
│   │   │   │   └── deployment_validator.py
│   │   │   ├── measurement/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── registry.py
│   │   │   │   ├── protocols.py
│   │   │   │   └── validator.py
│   │   │   ├── cli/
│   │   │   │   ├── __init__.py
│   │   │   │   ├── train.py
│   │   │   │   ├── evaluate.py
│   │   │   │   ├── deploy.py
│   │   │   │   ├── validate.py
│   │   │   │   └── extract_collision_contract.py
│   │   │   └── utils/
│   │   │       ├── __init__.py
│   │   │       ├── geometry.py
│   │   │       ├── paths.py
│   │   │       ├── logging.py
│   │   │       └── serialization.py
│   │   └── test/
│   │       ├── unit/
│   │       │   ├── test_config_compiler.py
│   │       │   ├── test_navigation_mode_matrix.py
│   │       │   ├── test_observation_schema.py
│   │       │   ├── test_lidar_angular_binning.py
│   │       │   ├── test_local_reference_sync.py
│   │       │   ├── test_lookahead_progress.py
│   │       │   ├── test_goal_settle.py
│   │       │   ├── test_command_envelope.py
│   │       │   ├── test_map_hash.py
│   │       │   ├── test_reward_contract.py
│   │       │   ├── test_termination_semantics.py
│   │       │   ├── test_contact_latch.py
│   │       │   └── test_safety_math.py
│   │       ├── integration/
│   │       │   ├── test_env_contract.py
│   │       │   ├── test_transactional_reset.py
│   │       │   ├── test_topic_qos_tf_contract.py
│   │       │   ├── test_ground_truth_isolation.py
│   │       │   ├── test_ekf_tf_ownership.py
│   │       │   ├── test_localization_monitor.py
│   │       │   ├── test_hybrid_navigation.py
│   │       │   ├── test_command_source_modes.py
│   │       │   ├── test_gazebo_contact.py
│   │       │   ├── test_checkpoint_resume.py
│   │       │   └── test_train_eval_isolation.py
│   │       ├── hil/
│   │       │   ├── test_mcu_watchdog.py
│   │       │   ├── test_mcu_reboot.py
│   │       │   └── test_estop_path.py
│   │       └── smoke/
│   │           ├── test_short_training.py
│   │           └── test_policy_inference.py
│   │
│   └── mecanum_nav_bringup/                      // Launch/config cấp toàn hệ thống
│       ├── package.xml
│       ├── CMakeLists.txt
│       ├── launch/
│       │   ├── local_training.launch.py
│       │   ├── mapping.launch.py
│       │   ├── nav2_baseline.launch.py
│       │   ├── hybrid_ai_local.launch.py
│       │   └── deploy_real.launch.py
│       ├── config/
│       │   ├── state_estimation/
│       │   │   ├── ekf_sim.yaml
│       │   │   └── ekf_real.yaml
│       │   ├── slam/slam_toolbox_mapping.yaml
│       │   ├── localization/amcl_omni.yaml
│       │   ├── nav2/
│       │   │   ├── common.yaml
│       │   │   ├── planner.yaml
│       │   │   ├── baseline_controller.yaml
│       │   │   └── costmaps.yaml
│       │   ├── safety/safety_runtime.yaml
│       │   └── modes/
│       │       ├── local_training.yaml
│       │       ├── mapping.yaml
│       │       ├── nav2_baseline.yaml
│       │       └── hybrid_ai_local.yaml
│       ├── test/
│       │   ├── test_local_training_launch.py
│       │   ├── test_mapping_launch.py
│       │   ├── test_nav2_baseline_launch.py
│       │   ├── test_hybrid_ai_local_launch.py
│       │   └── test_safety_fail_stop.py
│       └── rviz/navigation.rviz
├── build/                                         // Colcon sinh, không sửa tay
├── install/                                       // Colcon install space
└── log/                                           // Colcon logs

| Public component | File owner | Trách nhiệm duy nhất |
| --- | --- | --- |
| ConfigCompiler | config/compiler.py | Merge overlays, validate profile+mode, freeze, hash |
| MecanumBaseBridge | mecanum_base_bridge/bridge_node.py | STM32 transport, telemetry, final command transport |
| WheelOdometry | mecanum_base_bridge/wheel_odometry.py | Ticks FL/FR/RL/RR -> /wheel/odometry_raw; không TF |
| LocalizationMonitor | localization/monitor.py | Lifecycle, initial pose, TF, covariance, jump, map identity |
| HybridNavigationManager | navigation/hybrid_navigation_manager.py | Mission action, planning, replan, cancel, completion |
| Nav2PathProvider | navigation/nav2_path_provider.py | ComputePathToPose client and GlobalPathSnapshot |
| LookaheadSelector | navigation/lookahead.py | Arc-length progress -> synchronized LocalReference |
| ObservationEncoder | observations/encoder.py | Immutable 81-D float32 schema |
| SnapshotSynchronizer | ros/synchronizer.py | Scan/odom/reference epoch and timestamp alignment |
| ContactLatch | simulation/contact_latch.py | Active state + pending pulse |
| RewardManager | rewards/manager.py | Pure reward and breakdown |
| TerminationManager | termination/manager.py | Gym flags and precedence |
| RunSupervisor | training/run_supervisor.py | Process lifecycle and clean restart |
| CheckpointManager | training/checkpoint.py | Atomic rollout-boundary bundles |
| Evaluator | evaluation/evaluator.py | Independent sealed evaluation |
| CommandAdapter | deployment/command_adapters.py | Policy/Nav2/teleop -> CommandEnvelope |
| CommandValidator | safety/command_validator.py | Mode/source/epoch/generation/sequence/TTL/finite |
| CommandArbiter | safety/command_arbiter.py | Select one valid source or zero |
| SafetySupervisor | safety/supervisor.py | Hard envelope, collision shield, safe-stop decision |
| FinalTwistPublisher | safety/final_twist_publisher.py | Only publisher to /cmd_vel |

# 4. Typed immutable configuration

| Model | Nội dung |
| --- | --- |
| RuntimeProfile | SIM_TRAIN, SIM_EVAL, DEPLOY_SIM, DEPLOY_REAL |
| NavigationMode | LOCAL_TRAINING, MAPPING, NAV2_BASELINE, HYBRID_AI_LOCAL |
| FrameContractConfig | TF edges, authority and GT isolation |
| StateEstimationConfig | Wheel/IMU inputs, EKF owner, two_d_mode, covariance |
| LocalizationConfig | Provider, map identity, lifecycle, covariance/jump limits |
| NavigationConfig | Planner, replan, path validity, lookahead, goal settle |
| TopicContractConfig | Exact name/type/owner/QoS/frame/freshness |
| Observation/Action/RewardConfig | Versioned mathematical contracts |
| SafetyConfig | Command envelope, hard limits, shield, stop and E-stop |
| AcceptanceCriteriaConfig | Preregistered thresholds |
| ResolvedConfig | Deep-frozen result and canonical SHA-256 config_hash |

class ResolvedConfig(BaseModel):
    model_config = ConfigDict(frozen=True, extra="forbid")
    schema_version: Literal["3.0"]
    runtime_profile: RuntimeProfile
    navigation_mode: NavigationMode
    config_hash: str
    project: ProjectMetadata
    topics: tuple[TopicContractConfig, ...]
    frames: FrameContractConfig
    state_estimation: StateEstimationConfig
    localization: LocalizationConfig
    navigation: NavigationConfig
    observation: ObservationConfig
    action: ActionConfig
    reward: RewardConfig
    training: TrainingConfig
    evaluation: EvaluationConfig
    safety: SafetyConfig
    acceptance: AcceptanceCriteriaConfig

Merge order cố định: base -> runtime profile -> navigation mode -> approved experiment override -> approved measured hardware registry -> validation -> immutable ResolvedConfig -> config_hash.

Hash tính từ canonical JSON sau resolve, không chứa runtime timestamp. ResolvedConfig lưu cùng model, checkpoint, rollout manifest và report.

Runtime không mutate config; đổi config chỉ tại episode boundary và tạo hash/run mới.

# 5. Runtime profiles và navigation modes

| Field | sim_train | sim_eval | deploy_sim | deploy_real |
| --- | --- | --- | --- | --- |
| use_sim_time | true | true | true | false |
| allowed modes | LOCAL_TRAINING | NAV2_BASELINE; HYBRID_AI_LOCAL | MAPPING; NAV2_BASELINE; HYBRID_AI_LOCAL | MAPPING; NAV2_BASELINE; HYBRID_AI_LOCAL |
| policy pose/twist | internal SimulatedOdometryEmulator noisy pose/twist snapshot; no public topic | EKF/localization | EKF/localization | EKF/localization |
| evaluator/task oracle | GT allowed only in hidden emulator, task oracle, reset validation, reward/success/termination/evaluator | GT evaluator only | GT observer only | external metrology only |
| goal/reference | local goal | map mission -> LocalReference | map mission -> LocalReference | map mission -> LocalReference |
| LiDAR | sim RPLiDAR profile | sim RPLiDAR profile | sim RPLiDAR profile | YDLIDAR X3 profile |
| GT policy access | forbidden: GT topic, world/model pose, raw GT buffer/object | forbidden | forbidden | not present |
| randomization | episode-seeded bias/yaw bias, drift, velocity noise, delay queue, dropout; manifest required | fixed benchmark | measured/stress | none at runtime |
| reward | enabled | metrics only | disabled | disabled |

Policy/evaluator separation: Trong sim_eval/deploy_sim, process policy không được subscribe /ground_truth/odom. Evaluator riêng được phép dùng ground truth và T_world_map trong ScenarioManifest để tính metric; transform này không được publish vào TF hoặc chuyển cho policy.

# 6. Public protocols

class RobotRuntimeAdapter(Protocol):
    def reset(self, request: ResetRequest) -> ResetReceipt: ...
    def apply_action(self, command: VelocityCommand) -> ActionReceipt: ...
    def wait_transition_snapshot(self, after: ActionReceipt) -> SensorSnapshot: ...
    def stop(self) -> StopReceipt: ...
    def close(self) -> None: ...

class PoseProvider(Protocol):
    def pose(self, target_frame: str, at_ros_ns: int) -> Pose2DStamped: ...

class ContactEventSource(Protocol):
    def consume_for_transition(self, reset_epoch: int) -> ContactState: ...

class SafetyEvaluator(Protocol):
    def evaluate(self, state: SafetyStateSnapshot,
                 command: CommandEnvelope) -> SafetyDecision: ...

Protocol không để kiểu Any ở public boundary; exception taxonomy là ConfigError, SensorContractError, SyncTimeout, ResetFailure, SimulatorFailure và SafetyFault.

SimulatorAdapter và RealRobotAdapter implement cùng runtime contract nhưng reset capability khác; deploy_real không giả định teleport/reset world.

# 7. Immutable transition dataclasses

@dataclass(frozen=True)
class FrozenArray:
    value: NDArray[np.float32]  # copied, contiguous, writeable=False

@dataclass(frozen=True)
class SensorSnapshot:
    reset_epoch: int
    runtime_generation: int
    scan_ros_ns: int
    odom_ros_ns: int
    scan: LaserGeometrySnapshot
    pose: Pose2DStamped
    twist_body: FrozenArray
    contact: ContactState
    diagnostics: SnapshotDiagnostics

@dataclass(frozen=True)
class TransitionContext:
    previous: SensorSnapshot
    current: SensorSnapshot
    task: TaskState
    raw_action: FrozenArray
    decoded_command: VelocityCommand
    issued_command: VelocityCommand
    previous_issued_command: VelocityCommand
    dt_sim_s: float

Mọi ndarray được np.array(source, dtype=np.float32, copy=True, order='C') rồi setflags(write=False).

Callback cache không chia sẻ mutable memory với snapshot. Test dùng np.shares_memory và mutation attempts.

TransitionContext chỉ được tạo khi previous/current có cùng reset_epoch và runtime_generation đang active, timestamp mới hơn action barrier và scan/odom đồng bộ.

# 8. SafetyStateSnapshot

@dataclass(frozen=True)
class LaserGeometrySnapshot:
    ranges: FrozenArray
    angle_min: float
    angle_increment: float
    range_min: float
    range_max: float
    frame_id: str
    stamp_ros_ns: int
    received_steady_ns: int
    T_base_lidar: Transform2DSnapshot

@dataclass(frozen=True)
class SafetyStateSnapshot:
    reset_epoch: int
    runtime_generation: int
    scan: LaserGeometrySnapshot
    measured_twist: FrozenArray
    odom_stamp_ros_ns: int
    odom_received_steady_ns: int
    localization_health: HealthState
    navigation_state: NavigationStateCode
    navigation_source_instance_id: int
    navigation_sequence: int
    navigation_state_fresh: bool
    path_valid: bool
    motion_permitted: bool
    navigation_state_age_ms: int
    command_source_heartbeat: HeartbeatState
    mcu_heartbeat: HeartbeatState
    estop_state: EStopState
    reset_in_progress: bool
    diagnostics: SnapshotDiagnostics

Trong HYBRID_AI_LOCAL, SafetySensorNode nhận NavigationState và ghi steady receive time cục bộ. SafetySupervisor chỉ cho phép chuyển động khi state còn mới, epoch/generation/instance/sequence hợp lệ, path_valid=true và motion_permitted=true; đây vẫn chỉ là một phần của toàn bộ safety envelope.

SafetySensorNode fail-safe nếu scan/odom/TF stale, TF thiếu, generation đổi, reset đang chạy, frame sai, beam count sai hoặc angle metadata không hợp lệ.

NaN, -Inf, zero, negative là invalid; +Inf/no-return được phép trong raw LaserScan nhưng phải chuyển thành range_max trước geometry check và ghi ratio riêng.

Heartbeat/command TTL đo bằng steady clock; scan/odom header dùng ROS clock cho alignment. Không so hai clock bằng phép trừ trực tiếp.

# 9. ROS runtime và threading

| Process/node | Executor/callback group | Quy tắc |
| --- | --- | --- |
| training_env_process | MultiThreadedExecutor; sensor reentrant; reset/action mutually exclusive | Một env/một ROS context/một Gazebo instance |
| evaluator_process | Riêng training | Không dùng object/RNG/normalizer update của train |
| policy_process | Single/MultiThreaded | Chỉ inference; publish /mecanum/cmd_policy |
| safety_runtime_process | Dedicated executor | Adapters + arbiter + sensor + supervisor + final publisher |
| RunSupervisor | Non-ROS parent process | Spawn/terminate/restart, diagnostic and checkpoint selection |

Không spin_until_future_complete lồng trong callback. Service futures được đợi bởi orchestration thread với steady deadline.

Callbacks chỉ validate/copy/enqueue; không tính reward, không block reset và không gọi Gym API.

Safety process độc lập policy/Gym để policy treo không chặn safe stop.

# 10. Clock domains

| Clock | Dùng cho | Không dùng cho |
| --- | --- | --- |
| ROS time | Sensor stamps, simulation dt, TF lookup, episode sim-time | Watchdog/TTL/wall-time recovery deadline |
| steady monotonic | Receive age, command TTL, heartbeat, waits, restart timeout | TF/sensor timestamp comparison |
| system/UTC | Human logs, artifact creation time | Control or freshness |

ClockEpochTracker tăng runtime_generation khi /clock lùi, simulator restart hoặc reset transaction phát epoch mới.

Message mang epoch/generation không đúng bị loại trước synchronizer; future-dated sensor > configured tolerance là SensorContractError.

Control period baseline simulation 0.10 s theo ROS time; deadline chờ sử dụng steady time để không treo khi Gazebo pause.

# 11. Topic contract

| Topic | Exact type | Publisher được phép | Subscriber/owner | Modes/profiles |
| --- | --- | --- | --- | --- |
| /scan | sensor_msgs/msg/LaserScan | ros_gz_bridge or YDLIDAR driver | ObservationCollector; SafetySensorNode; localization | all |
| /ground_truth/odom | nav_msgs/msg/Odometry | Gazebo bridge | SimulatedOdometryEmulator internal; TrainingTaskOracle; reset transaction/validation; reward/success/termination; evaluator. Never PolicyNode/ObservationEncoder/policy runtime. | sim only; hidden from policy |
| /wheel/odometry_raw | nav_msgs/msg/Odometry | mecanum_base_bridge or sim wheel adapter | robot_localization EKF | integration/deploy |
| /imu/data | sensor_msgs/msg/Imu | Pi IMU driver or sim adapter | robot_localization EKF | integration/deploy |
| /odometry/filtered | nav_msgs/msg/Odometry | robot_localization EKF | AMCL/Nav2/Safety/Observation | integration/deploy |
| /map | nav_msgs/msg/OccupancyGrid | slam_toolbox in MAPPING; Map Server in navigation | AMCL/global costmap | mapping/navigation |
| /amcl_pose | geometry_msgs/msg/PoseWithCovarianceStamped | AMCL | LocalizationMonitor | navigation |
| /mecanum/contacts | ros_gz_interfaces/msg/Contacts | ros_gz_bridge | ContactLatch | sim train/eval |
| /cmd_vel_nav2_raw | geometry_msgs/msg/Twist | Nav2 Controller Server remapped cmd_vel | Nav2CommandAdapter | NAV2_BASELINE |
| /cmd_vel_teleop_raw | geometry_msgs/msg/Twist | Teleop node | TeleopCommandAdapter | MAPPING |
| /mecanum/cmd_policy | mecanum_nav_rl_interfaces/msg/CommandEnvelope | PolicyNode/PolicyCommandAdapter | SafetySupervisor | LOCAL_TRAINING/HYBRID |
| /mecanum/cmd_nav2 | mecanum_nav_rl_interfaces/msg/CommandEnvelope | Nav2CommandAdapter | SafetySupervisor | NAV2_BASELINE |
| /mecanum/cmd_teleop | mecanum_nav_rl_interfaces/msg/CommandEnvelope | TeleopCommandAdapter | SafetySupervisor | MAPPING |
| /cmd_vel | geometry_msgs/msg/Twist | FinalTwistPublisher only | Gazebo drive/mecanum_base_bridge | all control modes |
| /mecanum/heartbeat | mecanum_nav_rl_interfaces/msg/Heartbeat | PolicyNode; MCU bridge | SafetySensorNode | integration/deploy |
| /mecanum/navigation_state | mecanum_nav_rl_interfaces/msg/NavigationState | HybridNavigationManager | SafetySupervisor (machine input); UI/recorder | HYBRID_AI_LOCAL |
| /mecanum/safety_state | mecanum_nav_rl_interfaces/msg/SafetyState | SafetySupervisor | UI/logger | all control modes |
| /diagnostics | diagnostic_msgs/msg/DiagnosticArray | system nodes | aggregator/logger | all |
| /clock | rosgraph_msgs/msg/Clock | ros_gz_bridge only | use_sim_time nodes | sim |
| /tf; /tf_static | tf2_msgs/msg/TFMessage | declared authorities only | tf2 buffers | all |

| Topic | Stamp/frame contract | Stale/missing behavior |
| --- | --- | --- |
| /scan | ROS stamp; profile-specific lidar frame/FOV/range | reject snapshot or safe stop |
| /ground_truth/odom | header=world; child=base_link; never TF | oracle/evaluator fail only |
| /wheel/odometry_raw | header=odom; child=base_link; finite covariance; no TF | EKF unhealthy and safe stop |
| /odometry/filtered | ROS stamp; header=odom; child=base_link | reject/safe stop |
| /map | frame=map; approved map_id and map_manifest hash | startup inhibit |
| /cmd_vel_nav2_raw | unstamped Twist; local steady receipt authoritative | adapter rejects stale; zero fallback |
| /cmd_vel_teleop_raw | unstamped Twist; deadman + local steady receipt | adapter rejects stale; zero fallback |
| CommandEnvelope topics | header=base_link; epoch/generation/instance/sequence | reject; zero fallback |
| /cmd_vel | final actuator Twist | independent zero timer + MCU watchdog |
| heartbeat | sequence/epoch; steady receipt authoritative | zero/fault |
| navigation_state | frame=map; typed state/error; instance/sequence; epoch/generation; path_valid; motion_permitted | HYBRID: SAFE_STOP on missing/stale/mismatch/permit=false |
| safety_state | frame=base_link; ages embedded | last state for observers |

# 12. QoS contract

| QoS ID | Reliability | Durability | History/depth | Ứng dụng |
| --- | --- | --- | --- | --- |
| Q_SCAN | BEST_EFFORT | VOLATILE | KEEP_LAST/5 | LiDAR |
| Q_ODOM_RAW | RELIABLE | VOLATILE | KEEP_LAST/10 | wheel odometry |
| Q_ODOM_FILTERED | RELIABLE | VOLATILE | KEEP_LAST/10 | EKF output |
| Q_MAP | RELIABLE | TRANSIENT_LOCAL | KEEP_LAST/1 | occupancy map |
| Q_CONTACT | RELIABLE | VOLATILE | KEEP_LAST/50 | Gazebo contacts |
| Q_COMMAND | RELIABLE | VOLATILE | KEEP_LAST/1 | raw/enveloped commands |
| Q_HEARTBEAT | RELIABLE | VOLATILE | KEEP_LAST/10 | policy/MCU health |
| Q_NAVIGATION_STATE | RELIABLE | VOLATILE | KEEP_LAST/1 | hybrid safety input + runtime state |
| Q_SAFETY_STATE | RELIABLE | VOLATILE | KEEP_LAST/1 | safety runtime state |
| Q_DIAGNOSTICS | RELIABLE | VOLATILE | KEEP_LAST/10 | human diagnostics |
| Q_CLOCK | BEST_EFFORT | VOLATILE | KEEP_LAST/1 | simulation clock |
| Q_TF / Q_TF_STATIC | RELIABLE | VOLATILE / TRANSIENT_LOCAL | KEEP_LAST/100 / 1 | TF |

Ánh xạ bắt buộc: scan->Q_SCAN; wheel odometry->Q_ODOM_RAW; filtered odometry->Q_ODOM_FILTERED; contacts->Q_CONTACT; raw và enveloped commands->Q_COMMAND; heartbeat->Q_HEARTBEAT; navigation_state->Q_NAVIGATION_STATE; safety_state->Q_SAFETY_STATE; diagnostics->Q_DIAGNOSTICS; clock->Q_CLOCK.

Freshness/TTL do application steady-time guard, không dựa vào DDS Deadline. Đây là lựa chọn cố ý để cùng semantics trên ros_gz và micro-ROS.

StartupQoSProbe đọc endpoint offered/requested policies; mismatch trả lỗi gồm topic, endpoint GID, offered và required QoS. Required train/eval topic fail-fast; deploy chuyển SAFE_STOP.

# 13. TF/frame contract theo profile

| Profile | Pose/goal contract | TF edges bắt buộc | Authority duy nhất |
| --- | --- | --- | --- |
| sim_train | internal noisy snapshot/local goal; GT hidden emulator + oracle separate | base_link->sim_lidar; no new odom->base_link publisher in MVP | robot_state_publisher; sim_train odom->base_link authority deferred to reconciliation |
| sim_eval | map localization/mission | map->odom; odom->base_link; base_link->sim_lidar | AMCL; EKF; robot_state_publisher |
| deploy_sim | map localization/mission | map->odom; odom->base_link; base_link->sim_lidar | AMCL; EKF; robot_state_publisher |
| deploy_real | map localization/mission | map->odom; odom->base_link; base_link->real_lidar | AMCL; EKF; robot_state_publisher |

| Profile | Policy pose source | Evaluator/oracle source | Ground-truth isolation |
| --- | --- | --- | --- |
| sim_train | internal SimulatedOdometryEmulator noisy snapshot | /ground_truth/odom only for hidden emulator, oracle, reset validation, reward/success/termination/evaluator | not subscribed by PolicyNode/ObservationEncoder/policy runtime; no world/model pose or raw GT object |
| sim_eval | TF map->base_link | /ground_truth/odom + manifest transform | evaluator process only |
| deploy_sim | TF map->base_link | optional GT observer | policy namespace/process cannot subscribe |
| deploy_real | TF map->base_link | external metrology only | no world/GT dependency |

/ground_truth/odom có header.frame_id=world nhưng chỉ là pose message; GroundTruthPoseProvider không broadcast world->base_link.

sim_train dùng output noisy pose/twist nội bộ của SimulatedOdometryEmulator cho local goal; ground truth chỉ là input ẩn của emulator và task oracle. MVP không tạo public odometry topic hay TF publisher mới và không tạo parent thứ hai của base_link.

LiDAR frame và geometry được khóa theo profile: simulation dùng frame từ mô hình RPLiDAR A1M8; deploy_real lấy đúng header.frame_id của YDLIDAR X3. Mọi beam được gom theo góc vào 72 sector; không dùng alias im lặng.

# 14. TF failure policy

| Điều kiện | Train/eval | Deploy |
| --- | --- | --- |
| Missing required edge | Startup fail hoặc infrastructure exception | SAFE_STOP + diagnostic |
| Multiple authorities/same child parents | Startup fail | Startup inhibit / SAFE_STOP |
| TF age > configured steady/ROS bound | Reject snapshot | SAFE_STOP |
| Future-dated > tolerance | SensorContractError | SAFE_STOP + clock fault |
| Frame ID mismatch | Fail-fast; không tự sửa | SAFE_STOP |

TfContractValidator thu thập all_frames_as_yaml, endpoint/node ownership và lookup ở timestamp sensor; lỗi trả code TF001..TF005 cùng edge/authority/age.

Static TF base_link->lidar_frame phải khớp URDF của từng profile và do robot_state_publisher sở hữu duy nhất; deploy_real dùng đúng extrinsic đã đo của YDLIDAR X3.

# 15. Gazebo collision instrumentation

Ground-truth collision trong sim_train/sim_eval là REQUIRED_BUILD_INPUT được sinh từ expanded robot description; không giả định `/mecanum/contacts` tự tồn tại.

Build tool expand Xacro -> URDF, chạy `gz sdf -p`/inspect SDF, liệt kê fully scoped collision names, đối chiếu body/wheel geometry, rồi sinh contact sensor/bridge overlay.

Resolved profile chứa exact collision names và checksum của expanded description; còn placeholder collision-name chung hoặc mismatch checksum là build/startup error.

Startup probe xác minh Gazebo topic, `gz.msgs.Contacts`, ROS bridge `ros_gz_interfaces/msg/Contacts`, Q_CONTACT và controlled-contact fixture có message.

LiDAR min-range chỉ tạo `collision_estimate`; không được tính là ground-truth collision trong training/evaluation report.

extract_collision_contract \
  --xacro <REQUIRED_BUILD_INPUT.robot_xacro> \
  --body-links <REQUIRED_BUILD_INPUT.monitored_links> \
  --out generated/contact_contract.yaml

validate_contact_contract:
  expanded_description_sha256 == resolved_config.robot_description_hash
  every monitored collision exists exactly once
  controlled contact produces at least one valid Contacts message

# 16. Contact latch state machine

state = {reset_epoch, contact_active, collision_pulse_pending,
         last_valid_ros_ns, last_receive_steady_ns}

VALID_CONTACT(epoch, contacts_nonempty=True):
  if epoch != current_epoch: DROP
  contact_active = True
  collision_pulse_pending = True

VALID_CONTACT(epoch, contacts_nonempty=False):
  if epoch != current_epoch: DROP
  contact_active = False
  # pending pulse is NOT cleared

CONSUME_TRANSITION(epoch):
  collision_seen = contact_active OR collision_pulse_pending
  collision_pulse_pending = False
  # contact_active unchanged

RESET_BEGIN(new_epoch):
  switch epoch atomically; clear active/pending/stamps; reject old epoch forever

| Trước | Sự kiện | Sau | Transition thấy collision? |
| --- | --- | --- | --- |
| F,F | contact true | T,T | Có |
| T,T | consume | T,F | Có; contact vẫn active |
| T,F | contact false | F,F | Không ở transition sau |
| F,F | true rồi false trước tick | F,T | Có; pulse được giữ |
| bất kỳ | reset epoch mới | F,F/new epoch | Không nhận message epoch cũ |

Contact silence không chứng minh đã rời va chạm; chỉ message hợp lệ empty/no matching contacts mới xóa contact_active.

Controlled-contact fixture phải xác nhận sensor phát explicit empty state sau separation. Nếu plugin chỉ phát khi có contact, build phải thêm ContactStatePublisher định kỳ từ Gazebo state; không được dùng silence-timeout để giả định contact=false.

Test bắt buộc: sustained contact qua nhiều tick, pulse ngắn, false sau consume, reset trong contact, delayed old-epoch true/false và duplicate message.

# 17. Synchronization và stale protection

accept buffered scan and odom only when:
  buffer_metadata.runtime_generation == current_generation
  buffer_metadata.reset_epoch == current_reset_epoch
  stamp_ros_ns > action_receipt.barrier_ros_ns
  stamp_ros_ns > reset_receipt.barrier_ros_ns
  abs(scan_stamp - odom_stamp) <= sync_tolerance
  receive_age_steady <= sensor_timeout
  contract(frame, shape, angular metadata, finite policy) passes

reset_epoch/runtime_generation are internal cache metadata; LaserScan and Odometry messages are not assumed to carry those fields.

SnapshotSynchronizer chỉ ghép periodic scan+odom. Contact là latched event; heartbeat/safety health là latest-state với steady freshness.

Deque có bounded size, ordered insert policy và counters cho out-of-order, duplicate, old epoch, stale receive và sync miss.

Timeout train/eval ném SyncTimeout trước khi tạo transition; deployment SafetySensorNode chuyển SAFE_STOP.

Clock rollback làm invalidate toàn bộ buffers và tăng generation; timestamp mới có cùng số cũ vẫn không vượt generation barrier.

# 18. Transactional reset

Acquire ResetManager lock; đặt reset_in_progress=true và vô hiệu command sources.

FinalTwistPublisher gửi zero; simulation chờ acknowledgement/measured near-zero theo profile timeout.

Tăng reset_epoch trước mutation; clear synchronizer buffers và contact latch atomically.

Pause/teleport/reset entity và task theo SimulatorAdapter; seed TaskManager bằng episode seed lineage.

Reset reward units, termination windows, observation history, action smoother và previous issued command.

Unpause; đặt ROS-time reset barrier sau simulator acknowledgement.

Đợi scan+odom fresh đúng epoch/generation và TF contract; xác minh contact topic healthy.

Encode observation0, xác minh shape/dtype/bounds; chỉ sau đó reset_in_progress=false và cho phép control.

Bất kỳ bước nào fail: giữ zero/inhibit, phát ResetFailure code, đóng env; không trả observation giả.

RealRobotAdapter reset episode không teleport; nó yêu cầu operator/task-start handshake và pose/health preconditions.

# 19. Transition và Gymnasium contract

def step(raw_action):
    command = action_manager.process(raw_action)
    receipt = adapter.apply_action(command)
    snapshot = adapter.wait_transition_snapshot(after=receipt)
    context = transition_builder.build(previous, snapshot, task,
                                       raw_action, previous_command, command)
    reward, reward_terms = reward_manager.compute(context)
    terminated, truncated, reason = termination_manager.evaluate(context)
    observation = observation_encoder.encode(context.current, task, command)
    return observation, reward, terminated, truncated, info_builder.build(...)

`reset()` gọi `super().reset(seed=seed)` và trả `(observation, info)`; `step()` luôn five-tuple Gymnasium.

Một transition chỉ commit sau khi observation/reward/termination/info đều hợp lệ; exception trước return nghĩa là không có transition.

SB3 VecEnv compatibility/terminal_observation nằm trong EnvFactory wrapper, không làm sai Base Env semantics.

# 20. Exact observation contract

Schema `obs-v3-l72-g3-t3-c3-f32`, shape `(81,)`, dtype `float32`. Thứ tự cố định: 72 LiDAR sectors; goal distance/sin/cos; measured body twist vx/vy/wz; previous issued command vx/vy/wz.

| Input ray | Phân loại | Encode |
| --- | --- | --- |
| finite within profile range | valid | clip; angular-sector min; normalize by profile range_max |
| +Inf or above range_max | no_return | range_max; increment no_return ratio |
| NaN, -Inf, <=0 | invalid | range_max; increment invalid ratio |
| finite below range_min | invalid near-field | range_min; increment invalid ratio |

Raw LaserScan được kiểm tra theo profile về frame, FOV, angle_increment, range và freshness. RPLiDAR A1M8 mô phỏng và YDLIDAR X3 thật được angular-binning vào cùng 72 sector; không reshape theo beam index và không giả định raw beam count giống nhau.

Goal features lấy từ LocalReference đã đồng bộ SensorSnapshot: clip(distance_m/goal_distance_scale_m,0,1), sin(bearing_rad), cos(bearing_rad), luôn float32.

Previous issued command là lệnh cuối SafetySupervisor đã chấp nhận/giới hạn và publish; không được gọi là actuator-applied khi chưa có acknowledgement. Vận tốc thực tế lấy từ measured_twist.

Schema hash gồm order, lidar rules, limits, goal cap, control period, issued-command definition và normalization.

# 21. Exact action và command contract

policy_action: Box(low=-1, high=1, shape=(3,), dtype=float32)
order: [vx_body, vy_body, wz]

normalized action -> decoded desired velocity -> CommandEnvelope SOURCE_POLICY
SafetySupervisor: epoch/generation/source/sequence/TTL/finite/hard-limit checks
SafetySupervisor -> final limited geometry_msgs/Twist -> /cmd_vel

ROS CommandEnvelope fields:
source, sequence, reset_epoch, runtime_generation, source_instance_id, twist
Header stamp is ROS time; safety TTL uses local steady receive time.

Simulation limits are versioned engineering baselines only. Real vx/vy/wz, acceleration, jerk and braking envelopes remain TBD_MEASURED and are never inherited silently from simulation.

/mecanum/cmd_policy, /mecanum/cmd_nav2 và /mecanum/cmd_teleop là các CommandEnvelope sau adapter. Nav2 Controller Server phải remap cmd_vel sang /cmd_vel_nav2_raw; teleop phải xuất /cmd_vel_teleop_raw. Chỉ FinalTwistPublisher được publish geometry_msgs/Twist trên /cmd_vel.

Nav2CommandAdapter subscribe /cmd_vel_nav2_raw; TeleopCommandAdapter subscribe /cmd_vel_teleop_raw. Adapter ghi local steady receive time, gắn epoch/generation/source_instance_id và xóa cache khi reset, đổi mode hoặc source restart.

# 22. Exact reward contract

delta_d = previous.goal_distance - current.goal_distance
r_progress  = clip(2.00 * delta_d, -1.00, 1.00)
r_time      = -0.10 * dt_sim_s
r_clearance = -0.20 * max(0, (d_safe-c)/d_safe) * dt_sim_s/0.10
du = normalized_applied - normalized_previous_applied
r_smooth    = clip(-0.005 * dot(du,du)/max(dt_sim_s,1e-6), -1.0, 0.0)
r_terminal  = +15 goal | -15 collision | -3 stuck | 0 otherwise
r_total     = sum(terms); abs(r_total - logged_sum) <= 1e-6

Các hệ số là `reward-v3-baseline` SIM_BASELINE, không phải optimum; ablation/multi-seed quyết định thay đổi schema.

RewardManager chỉ nhận TransitionContext và không đọc callback/ROS/global mutable state.

Collision terminal thắng goal nếu cùng transition. Infrastructure error không có reward.

# 23. Termination precedence

| Priority | Reason | terminated | truncated |
| --- | --- | --- | --- |
| 1 | collision_contact | true | false |
| 2 | goal_reached | true | false |
| 3 | out_of_bounds | true | false |
| 4 | stuck | true | false |
| 5 | max_steps/max_sim_time | false | true |
| N/A | sensor/TF/simulator/ROS infrastructure | no transition; raise | no transition; raise |

Terminal MDP event thắng external timeout đồng thời. Info có reason, is_success, step, elapsed sim time, reset_epoch và hashes.

Stuck window/threshold là versioned config; real values cần TBD_MEASURED/TBD_PROJECT_ACCEPTANCE tùy mục đích.

# 24. Task, scenario và seed

master_seed -> env_rng | task_rng | randomization_rng | model_rng
evaluation_rng is independent and comes from sealed scenario manifest

Manifest: scenario_id, family, split, world hash, start/goal, T_world_map nếu cần, obstacle seed, difficulty và checksum.

Train/validation/final_test split theo scenario family; final_test sealed đến khi model/config/acceptance preregistration khóa.

GoalSampler xác minh bounds, footprint clearance, connected/reachable region và start-goal distance.

Mỗi episode ghi seed lineage và realized randomization. Curriculum chỉ dùng validation, không đọc final_test.

# 25. PPO infrastructure failure recovery

env infrastructure exception
  -> collect_rollouts exits without fabricated transition
  -> discard entire uncommitted partial rollout
  -> diagnostic bundle + failed attempt record
  -> close trainer/env/ROS/Gazebo processes
  -> RunSupervisor restarts clean processes within budget
  -> load latest COMPLETE rollout-boundary checkpoint

Không mutate SB3 RolloutBuffer để giữ partial samples; không biến failure thành collision/timeout penalty.

Restart budget là config. Vượt budget làm run FAILED, không tự động tiếp tục vô hạn.

Resume được gọi statistically equivalent, không hứa bitwise identical do Gazebo/ROS scheduling.

# 26. Checkpoint và resume

Bundle gồm model/optimizer, VecNormalize nếu dùng, ResolvedConfig, config/schema hashes, manifest/checksums, committed timestep, callbacks/curriculum và RNG Python/NumPy/Torch CPU-CUDA/env/task/randomization.

Chỉ save tại `n_steps*n_envs` rollout boundary; V2.3 baseline n_envs=1. Mid-rollout checkpoint bị cấm.

Write temp -> fsync files -> fsync directory -> atomic rename -> marker COMPLETE cuối cùng.

Loader chọn newest compatible COMPLETE bundle; corrupted/incompatible bundle bị quarantine với structured error, không fallback im lặng.

Model load bị từ chối nếu observation/action/reward/config/protocol hash không khớp target profile.

# 27. Evaluation và thống kê

policy_success_rate = successes / valid_episodes
end_to_end_success_rate = successes / attempted_episodes
infrastructure_failure_rate = infrastructure_failures / attempted_episodes
SPL = mean(success_i * shortest_i / max(path_i, shortest_i))

| Nhóm | Metrics bắt buộc |
| --- | --- |
| Outcome | success, collision, timeout, stuck, out_of_bounds, reason |
| Efficiency | SPL, path length/efficiency, time-to-goal |
| Safety | min/mean clearance, intervention rate, stop/E-stop count |
| Motion | acceleration RMS, jerk RMS, angular oscillation, energy proxy |
| Tracking | command-vs-measured vx/vy/wz error |
| Reliability | attempted/valid, infra failures, restarts, replay failures |

Eval env/process/RNG/scenario/normalization độc lập training; deterministic policy; no learning/no normalizer update.

Báo per-seed, mean/std, 95% confidence interval và raw episode records. Paired comparison dùng cùng manifests/seeds khi hợp lệ.

Replay/infra failure vẫn ở attempted denominator; không loại im lặng để làm đẹp success rate.

# 28. AcceptanceCriteriaConfig và preregistration

class AcceptanceCriteriaConfig(BaseModel):
    model_config = ConfigDict(frozen=True, extra="forbid")
    schema_version: Literal["acceptance-v3"]
    min_training_seeds: int
    evaluation_seeds: int | ProjectAcceptanceToken
    episodes_per_scenario: int | ProjectAcceptanceToken
    min_success_rate: float | ProjectAcceptanceToken
    max_collision_rate: float | ProjectAcceptanceToken
    max_timeout_rate: float | ProjectAcceptanceToken
    max_stuck_rate: float | ProjectAcceptanceToken
    max_infrastructure_failure_rate: float | ProjectAcceptanceToken
    max_safety_intervention_rate: float | ProjectAcceptanceToken
    min_path_efficiency: float | ProjectAcceptanceToken
    max_time_to_goal_s: float | ProjectAcceptanceToken
    max_control_jerk_rms: float | ProjectAcceptanceToken
    confidence_level: float
    baseline_comparison_rule: str | ProjectAcceptanceToken
    approved_by: str | None
    approved_at: str | None
    preregistration_hash: str | None

Methodological minimum training seeds được khóa >=3; final claim nên 5 nếu ngân sách cho phép. Outcome thresholds còn lại là TBD_PROJECT_ACCEPTANCE.

Owner: Project Principal Investigator. Deadline: trước khi mở sealed final_test hoặc xem kết quả dùng cho claim.

Sau approval, artifact canonical được hash và read-only; thay đổi tạo preregistration version mới và làm mất tư cách confirmatory của kết quả đã xem.

Gate 2/3 fail nếu token chưa resolve hoặc thiếu approver/time/hash. Không gọi threshold dự án là chuẩn ROS/DRL phổ quát.

# 29. Domain randomization

| Profile | Bounds | Mục đích |
| --- | --- | --- |
| nominal | Verified simulator values | Baseline |
| mild | Pre-registered narrow engineering range | Sensitivity |
| measured | Empirical joint distributions từ M01-M12 VALID | Sim-to-real claim |
| stress | Outside measured, gắn nhãn extrapolation | Failure envelope |

Randomize LiDAR noise/dropout/latency, command delay/jitter, odom/localization noise, wheel response/slip, friction, payload/mass, battery proxy và obstacles.

Mỗi episode ghi realized parameter vector. Correlated parameters dùng joint model, không sample độc lập tùy tiện.

Không randomize để che TF/clock/physics bug. Measured profile chỉ dùng registry records VALID và matching hardware/firmware/calibration.

# 30. Deployment robustness

deploy_sim dùng đúng observation/action/source-adapter/safety path như deploy_real; policy process không được cấp ground truth.

Robustness report tách nominal/mild/measured/stress; báo absolute metric và retention. Retention khi nominal success=0 là undefined và Gate fail.

HIL inject latency, jitter, packet loss, LiDAR dropout, slip, localization degradation, MCU restart và DDS/node failure.

Staged real tests: wheels lifted -> tethered low speed -> controlled empty zone -> static obstacles -> operational envelope. Mỗi bước có stop criteria preregistered.

# 31. Command arbitration

Policy/Nav2/teleop -> source adapters -> CommandEnvelope ROS messages
  -> CommandValidator -> CommandArbiter -> SafetySupervisor
  -> FinalTwistPublisher -> /cmd_vel

hardware E-stop / critical fault > safe stop > allowed source for NavigationMode > zero fallback

| Source | Allowed mode | Validation | Handover |
| --- | --- | --- | --- |
| hardware E-stop/critical fault | all | latched hardware path | zero/disable immediately |
| teleop | MAPPING | deadman + envelope + TTL | controlled from measured twist |
| Nav2 | NAV2_BASELINE | epoch/generation/source instance/seq/TTL | same final safety envelope |
| policy | LOCAL_TRAINING; HYBRID_AI_LOCAL | epoch/generation/source instance/seq/TTL | same final safety envelope |
| zero fallback | all | always valid | selected on invalid/expired/no owner |

NavigationMode bảo đảm chỉ một nguồn bình thường hợp lệ: POLICY trong LOCAL_TRAINING/HYBRID_AI_LOCAL, TELEOP trong MAPPING, NAV2 trong NAV2_BASELINE. Wrong-mode, old epoch/generation, stale source instance, duplicate/out-of-order, non-finite, wrong-frame hoặc expired command đều bị reject.

SafetySupervisor là chủ sở hữu hard velocity/acceleration/jerk envelope và final stop. Controller có soft feasibility limits nhưng baseline đầu không chạy Nav2 Velocity Smoother. E-stop/critical fault không bị trì hoãn bởi normal ramp.

FinalTwistPublisher duy nhất publish /cmd_vel và có independent zero timer. GraphValidator phát hiện publisher khác; deploy inhibit.

CommandDecision log: source_id, sequence, source/receive stamps, raw, limited, final command và intervention reason.

# 32. Safety runtime

Kiểm tra physical/software E-stop và MCU fault latch.

Chọn CommandEnvelope theo mode/priority/lease; hết TTL chọn zero.

Kiểm tra runtime generation/reset state và scan/odom/TF/localization/heartbeat freshness.

Kiểm tra scan shape, geometry, frame, invalid/no-return ratios và finite measured twist.

Giới hạn velocity envelope theo hướng, payload, battery và surface profile.

Tính swept footprint/TTC; chọn PASS, LIMIT, SLOW, STOP hoặc E_STOP.

Áp acceleration/jerk limiter cho normal command; emergency dùng measured emergency braking/hardware path.

Publish Twist cuối và SafetyState/DiagnosticArray có reason codes.

Safety loop thuộc dedicated process/executor, chạy theo sensor/command events và independent timer; không bị giới hạn ở policy 10 Hz.

Mọi invalid/stale/missing input mặc định fail-safe. Chỉ explicit healthy state mới cho lệnh khác zero.

SafetyStateSnapshot là dữ liệu nội bộ giàu thông tin; ROS SafetyState.msg chỉ xuất trạng thái/ages/commands cần quan sát, không gửi toàn bộ scan lần hai.

HYBRID fail-stop: thiếu NavigationState, state quá TTL theo steady receive time, source instance thay đổi chưa handshake, sequence lùi/trùng, sai reset_epoch/runtime_generation, path_valid=false hoặc motion_permitted=false đều buộc chọn zero fallback. IDLE, PLANNING không có path dùng được, STOPPING, SUCCEEDED, CANCELED và FAILED luôn không cho phép chuyển động.

# 33. Holonomic swept-footprint collision shield

u = [vx, vy, wz]
for t in [0, horizon] with validated integration step:
    pose_t = integrate_SE2(u, t)
    footprint_t = transform(verified_footprint, pose_t)
swept_region = union(footprint_t) + uncertainty_inflation
TTC = first intersection(swept_region(t), lidar_obstacle_points)

TTC <= stop_ttc -> STOP/E_STOP
stop_ttc < TTC <= slow_ttc -> continuous scale
otherwise -> pass to normal dynamic limiter

Test forward, reverse, lateral, diagonal, pure rotation và translation+rotation gần góc footprint.

Footprint lấy từ measured/verified geometry, không phải robot point hoặc chỉ front rays.

Uncertainty inflation gồm LiDAR, TF/time alignment, localization và footprint error; giá trị real là TBD_MEASURED.

RPLiDAR 2D có blind plane; bumper/MCU/hardware E-stop là lớp bổ sung, không được shield phần mềm thay thế.

# 34. Stopping distance và physical E-stop

d_stop(direction, speed, payload, surface, battery) =
    speed * (latency_p99 + jitter_p99 + sensor_age_bound)
  + braking_distance_measured(direction, speed, payload, surface, battery)
  + sensor_localization_uncertainty_bound
  + engineering_reserve

physical E-stop button -> safety relay/MCU -> motor driver enable/power contactor
                      # independent of Linux, ROS 2, DDS and Python
auxiliary contact -> MCU/IO -> Heartbeat/Safety diagnostics only

Đo riêng forward/reverse/lateral/diagonal/yaw trên nhiều speed, payload, surface và battery conditions.

Stop/slow zones phải bao ngoài end-to-end measured stopping envelope; software reaction time đơn lẻ không đủ.

E-stop/fault latched, manual reset và không auto-resume. Fault injection phải chứng minh motor disable khi OS/DDS/safety node chết.

Tài liệu này thiết kế safety gates nhưng không tuyên bố functional safety certification.

# 35. ROS 2 packaging và build contract

| Package | Build type | Trách nhiệm/dependencies chính |
| --- | --- | --- |
| ROBOT_URDF_final_description | ament_cmake | URDF/Xacro, Gazebo models/world and static geometry |
| mecanum_nav_rl_interfaces | ament_cmake | Heartbeat, SafetyState, NavigationState, CommandEnvelope |
| mecanum_base_bridge | ament_python | STM32 transport, wheel odometry, final command transport |
| mecanum_nav_rl | ament_python | DRL, hybrid manager, navigation adapters, safety, train/eval |
| mecanum_nav_bringup | ament_cmake | System launch/config for EKF, SLAM, AMCL, Nav2 and safety |

Project metadata đã khóa: Ha Van Vu, havan.vu1302@gmail.com, Apache-2.0. Package mới chưa triển khai không được tuyên bố build-tested trước khi colcon/CI pass.

mecanum_nav_rl installs DRL assets; mecanum_nav_bringup owns system launch and EKF/SLAM/AMCL/Nav2/Safety configs; mecanum_base_bridge owns STM32 transport and /wheel/odometry_raw. Mutable maps/models/checkpoints/reports remain outside src.

ROS deps do package.xml/rosdep; Python ML deps do pinned lockfile/container. CI: rosdep install -> colcon build --symlink-install -> colcon test -> colcon test-result --verbose.

Mutable models/checkpoints/reports không cài vào package share và không nằm trong src; paths lấy từ ResolvedConfig.

# 36. Schema và startup validators

| Validator | Kiểm tra | Ví dụ code |
| --- | --- | --- |
| ConfigValidator | runtime profile + navigation mode; TBD/profile constraints | CFG001 |
| TopicContractValidator | name/type/owner/endpoint | TOP001 |
| QoSCompatibilityProbe | offered/requested compatibility | QOS001 |
| TfContractValidator | single authority; no GT TF; edge/frame/age | TF001 |
| StateEstimationValidator | wheel order/covariance/EKF two_d/TF owner | EST001 |
| LocalizationValidator | lifecycle/initial pose/covariance/jump/map | LOC001 |
| NavigationValidator | planner/controller mode ownership/path/reference | NAV001 |
| LaserContractValidator | profile geometry + angular binning | LID001 |
| MapArtifactValidator | approval/map_id/canonical hash/path confinement | MAP001 |
| CommandValidator | mode/source/epoch/generation/instance/seq/TTL/finite | CMD001 |
| ActuatorOwnershipValidator | only FinalTwistPublisher on /cmd_vel | CMD002 |
| ModelCompatibilityValidator | obs/action/reward/config/protocol hashes | MOD001 |

ValidationError:
  code: str
  severity: FATAL | SAFE_STOP | WARNING
  component: str
  expected: object
  actual: object
  profile: str
  remediation: str

ValidationReport:
  passed: bool
  errors: tuple[ValidationError, ...]
  resolved_config_hash: str

Validator không trả `False` đơn độc. Train/eval required-contract error là fatal; deploy sensor/runtime error sau startup là SAFE_STOP.

Observation dimension phải đúng 81; action Box/dtype/bounds; reward schema; checkpoint hash; collision instrumentation và sensor rate đều được kiểm tra tự động.

# 37. Test matrix

| Tầng | Bắt buộc |
| --- | --- |
| Unit | core/config; angular LiDAR; lookahead; goal settle; map hash; command envelope; wheel kinematics |
| Property/fuzz | NaN/Inf; timestamp skew; counter wrap; packet loss; old epoch/generation; random reset |
| ROS integration | types/QoS/TF ownership; EKF; localization health; action cancel/preempt; one /cmd_vel |
| Gazebo | contact; 500 resets; 10k commands; GT isolation; mapping/Nav2/hybrid launch |
| Training/evaluation | PPO smoke; multi-seed; checkpoint recovery; sealed final test; Nav2 baseline fairness |
| HIL/real | latency/jitter/loss/slip/localization/MCU reboot/watchdog/E-stop/stopping distance |

Per-commit: unit/schema/short launch smoke. Nightly: Gazebo endurance/fuzz/recovery. Hardware lane: HIL and E-stop fault injection.

Mỗi requirement ID trong traceability matrix phải có test ID; test evidence gồm config hash, commit, environment and artifact checksum.

ACR S1 required tests: policy observation accepts only emulator output; graph/config forbids PolicyNode/ObservationEncoder subscription to /ground_truth/odom; reset clears bias/drift/delay queue by episode seed; equal seed plus parameter manifest and input trace is reproducible; MecanumDrive /odom is not declared policy odometry.

# 38. Gate 1-4

| Gate | Entry/exit criteria |
| --- | --- |
| G1 Correctness/Reliability | All contract tests pass; 500 resets; 10k random actions; zero invalid transition; infra rate meets preregistered acceptance |
| G2 Learning Validity | >=3 train seeds; sealed splits; deterministic independent eval; approved acceptance config; raw metrics + CI |
| G3 Sim-to-Real Robustness | Measured profile provenance VALID; HIL perturbations; absolute/retention metrics meet preregistered thresholds |
| G4 Deployment Safety | No TBD_MEASURED; footprint/stopping envelope VALID; HIL/staged tests pass; physical E-stop independent; no unsafe bypass |

Gate rule: Gate sau không bù Gate trước. Gate 2/3 không thể chạy confirmatory khi TBD_PROJECT_ACCEPTANCE chưa được phê duyệt; Gate 4 không thể pass khi còn TBD_MEASURED hoặc REQUIRED_BUILD_INPUT.

# 39. Measurement registry

| ID | Measurement | Method/statistic | Owner | Status |
| --- | --- | --- | --- | --- |
| M01 | velocity/accel envelope by direction | repeated trials; p95/p99/worst | Controls | TBD_MEASURED |
| M02 | sensor-to-actuator latency/jitter | timestamped E2E; p99/worst | Systems | TBD_MEASURED |
| M03 | braking distance by direction/condition | controlled stops; upper bound | Safety | TBD_MEASURED |
| M04 | LiDAR noise/dropout/range bias | calibration logs; distributions | Perception | TBD_MEASURED |
| M05 | localization uncertainty/loss | external GT; p95/p99/failure | Localization | TBD_MEASURED |
| M06 | wheel response/slip/traction | tracking trials; joint distributions | Controls | TBD_MEASURED |
| M07 | watchdog/safe-stop end-to-end | fault injection; worst | Safety | TBD_MEASURED |
| M08 | physical E-stop independence | OS/DDS/node failures | Hardware Safety | TBD_MEASURED |
| M09 | footprint/collision geometry | physical vs URDF inspection | Mechanical | TBD_MEASURED |
| M10 | payload/battery/surface envelope | stratified repeated trials | Systems | TBD_MEASURED |
| M11 | MCU heartbeat/restart behavior | fault injection; p99 recovery | Firmware | TBD_MEASURED |
| M12 | TF/sensor time alignment error | bag analysis; p99/worst | ROS Integration | TBD_MEASURED |

Record fields bắt buộc: owner, method, equipment/software, repetitions, p95/p99/worst as applicable, raw path, analysis commit, hardware/firmware, calibration, payload, surface, battery, date, checksum, reviewer và status.

Status chỉ `TBD_MEASURED | VALID | STALE | REJECTED`. ConfigCompiler chỉ resolve từ VALID record matching target hardware profile.

Hardware/firmware/mount/payload envelope thay đổi làm affected records STALE và khóa lại Gate 3/4.

# 40. Traceability matrix

| Req | Component/owner | Config | Test | Gate |
| --- | --- | --- | --- | --- |
| R-TF | EKF/AMCL/TfValidator | frames/state_estimation | T-TF | G1/G3 |
| R-EST | BaseBridge/EKF | wheel/ekf | T-EST | G1/G3 |
| R-LOC | LocalizationMonitor | localization | T-LOC | G1/G3 |
| R-NAV | HybridManager/Nav2 | navigation | T-NAV | G1/G2 |
| R-MAP | MapArtifactValidator | map manifest | T-MAP | G1/G3 |
| R-SYNC | Synchronizer/LocalReference | synchronization | T-SYNC | G1 |
| R-OBS | ObservationEncoder | observation/AgentSpec | T-OBS | G1/G2 |
| R-PPO | Trainer/Checkpoint | training | T-PPO | G1/G2 |
| R-EVAL | Evaluator/BaselineRunner | evaluation/acceptance | T-EVL | G2/G3 |
| R-CMD | Adapters/Validator/Arbiter | command_sources | T-CMD | G1/G4 |
| R-SAFE | SafetySupervisor/Shield | safety | T-SAF | G3/G4 |
| R-MEASURE | MeasurementRegistry | measurements | T-MSR | G3/G4 |

# 41. State estimation và wheel odometry

STM32 gửi cumulative encoder ticks theo thứ tự FL, FR, RL, RR. mecanum_base_bridge kiểm tra protocol_version, sequence, mcu_boot_id, reset_epoch và MCU monotonic time trước khi tính delta.

mcu_boot_id thay đổi khi MCU reboot; reset_epoch thay đổi khi runtime reset. Base bridge không tính delta xuyên qua boot/epoch và không broadcast TF.

robot_localization EKF fuse /wheel/odometry_raw và /imu/data, dùng two_d_mode=true, publish /odometry/filtered và là authority duy nhất odom->base_link trong integration/deployment profiles.

artifacts/hardware/<hardware_profile_id>/hardware_manifest.yaml là nguồn chuẩn duy nhất cho wheel order, dấu, bán kính bánh, wheelbase, track, encoder CPR và gear ratio. wheel_geometry.yaml và firmware kinematics_params.h được sinh hoặc xác minh từ manifest; bridge so sánh hardware_config_hash với hash firmware trước khi enable motor.

## 41.1 Simulated odometry và ground-truth isolation cho sim_train

sim_train canonical data flow: hidden Gazebo ground truth -> SimulatedOdometryEmulator nội bộ -> noisy pose/twist snapshot -> SnapshotSynchronizer -> ObservationEncoder -> PPO. MVP output là internal typed snapshot, không public nav_msgs/Odometry và không publish TF.

Noise MVP bắt buộc có position/yaw bias theo episode, random-walk drift, velocity noise, delay queue và dropout. Reset xóa state emulator theo episode seed; sampled parameter manifest phải được lưu để tái lập.

MecanumDrive /odom là legacy compatibility/wheel-visualization/odom path, không phải actuator chính thức hay policy odometry canonical. VelocityControl là intended model-motion actuator của legacy simulation; Gazebo actuator, /odom và TF reconciliation là deferred work.

sim_eval/deploy_sim target: encoder emulator -> simulated /wheel/odometry_raw equivalent -> EKF -> /odometry/filtered. deploy_real target: STM32 encoder ticks -> /wheel/odometry_raw -> EKF. EKF là odom->base_link authority duy nhất trong integration/deployment profiles.

# 42. Bốn chế độ vận hành

| NavigationMode | Luồng điều khiển | Thành phần không được chạy |
| --- | --- | --- |
| LOCAL_TRAINING | PPO -> CommandEnvelope -> Safety -> simulator | SLAM, AMCL, Nav2 |
| MAPPING | Teleop -> Envelope -> Safety -> robot | AMCL, PPO auto, Nav2 Controller |
| NAV2_BASELINE | BT/Planner/MPPI Omni -> raw Twist -> Adapter -> Envelope -> Safety | PPO controller, Velocity Smoother v1 |
| HYBRID_AI_LOCAL | Planner -> LocalReference -> PPO -> Envelope -> Safety | Nav2 Controller, FollowPath |

## Nav2 baseline controller contract

controller_server:
  ros__parameters:
    odom_topic: /odometry/filtered
    enable_stamped_cmd_vel: false
    min_x_velocity_threshold: 0.001
    min_y_velocity_threshold: 0.001
    min_theta_velocity_threshold: 0.001
    progress_checker_plugins: [progress_checker]
    goal_checker_plugins: [position_goal_checker]
    controller_plugins: [FollowPath]
    progress_checker.plugin: nav2_controller::SimpleProgressChecker
    position_goal_checker.plugin: nav2_controller::PositionGoalChecker
    FollowPath.plugin: nav2_mppi_controller::MPPIController
    FollowPath.motion_model: Omni
launch remap: cmd_vel -> /cmd_vel_nav2_raw
Velocity Smoother v1: disabled; hard limits remain owned by SafetySupervisor

# 43. Mapping và localization

MAPPING chạy slam_toolbox mapping với teleop qua SafetySupervisor. AMCL không chạy đồng thời. Map mới ở trạng thái draft cho tới khi lưu, hash, kiểm tra và phê duyệt.

Navigation V1 dùng Map Server + AMCL với robot_model_type=nav2_amcl::OmniMotionModel. SLAM Toolbox chỉ chạy trong MAPPING; không tuyên bố hoặc cấu hình backend SLAM localization trong NAV2_BASELINE/HYBRID_AI_LOCAL.

LocalizationMonitor không chỉ nhìn TF: nó kiểm tra lifecycle, initial pose, pose/covariance freshness, pose jump, map_id và map_content_hash.

# 44. HybridNavigationManager

Expose /hybrid_navigate_to_pose bằng nav2_msgs/action/NavigateToPose. Goal behavior_tree phải rỗng; hybrid manager không chạy BT do caller cung cấp.

HybridNavigationManager tăng source_instance_id mỗi lần process khởi động, tăng sequence trên từng bản tin và phát NavigationState định kỳ khi mission active. Ở REPLANNING chỉ được giữ motion_permitted=true nếu đường cũ vẫn hợp lệ, chưa hết hạn và đúng epoch/generation; còn lại phải false.

Manager gọi ComputePathToPose, quản lý goal/path IDs, timeout, replan, deviation, cancel/preemption, reset và mission completion; không publish velocity.

NavigationState.msg là machine-readable safety input trong HYBRID_AI_LOCAL, đồng thời phục vụ UI/recorder. Message mang state typed, source_instance_id, sequence, reset_epoch, runtime_generation, path_valid và motion_permitted. DiagnosticArray và detail chỉ để quan sát; machine logic không parse chuỗi.

# 45. GlobalPath và LocalReference

GlobalPathSnapshot mang goal_id, path_sequence, reset_epoch, runtime_generation, map_id, map_content_hash, ROS path stamp, steady receipt/expiry và immutable poses.

Lookahead chiếu robot trong cửa sổ quanh segment trước, theo dõi arc-length, cho phép backtracking nhỏ và không nhảy nhánh tại path giao nhau.

LocalReference mang source path stamp, transform/reference ROS stamps, steady expiry, point in base_link, distance và bearing. Observation builder kiểm tra skew với SensorSnapshot cùng epoch/generation.

normalized_distance = clip(local_reference.distance_m / goal_distance_scale_m, 0.0, 1.0)
goal_features = float32([normalized_distance, sin(bearing_rad), cos(bearing_rad)])
abs(local_reference.reference_stamp_ros_ns - snapshot.odom_stamp_ns)
    <= max_reference_snapshot_skew_ns

# 46. Mission completion position only

Phiên bản đầu khóa require_goal_orientation=false. Final yaw không phải success criterion và Nav2 baseline dùng position-only goal checker để so sánh công bằng.

Mission thành công khi position tolerance, hypot(vx,vy), abs(wz) và localization health cùng đạt liên tục trong goal_settle_time_s. Đến LocalReference không phải đến MissionGoal.

# 47. Command envelope và reset isolation

CommandEnvelope.msg mang source, uint64 sequence, reset_epoch, runtime_generation, source_instance_id và Twist trong base_link. Header dùng ROS time; SafetySupervisor ghi steady receipt time cục bộ để tính TTL.

Safety chỉ chấp nhận source đúng NavigationMode và loại wrong mode, old epoch/generation/source instance, duplicate/backward sequence, stale, non-finite hoặc out-of-envelope command.

On reset, source restart or mode handover, cached commands and sequence state are invalidated; zero is selected until a fresh envelope passes all checks.

# 48. Map artifact identity

artifacts/maps/<map_id> là nguồn map duy nhất và chứa map.yaml, map.pgm, map_manifest.json, metadata.yaml cùng pose graph tùy chọn. Bringup nhận map_path, expected_map_id và expected_map_content_hash; không giữ bản map chỉnh sửa thứ hai.

map_content_hash là SHA-256 của canonical JSON trong map_manifest.json, gồm role, relative filename, size và SHA-256 của raw map.yaml và map.pgm. pose_graph_manifest.json dùng schema/hash riêng; metadata.yaml không tự nằm trong hash mà nó lưu.

Mapping creates draft artifacts and hashes after save. Navigation requires approval_status=approved. Pose graph uses a separate versioned manifest hash.

map_hash_manifest = {
  schema: mecanum_map_hash/v1,
  files: [
    {role: map_yaml, name, size_bytes, sha256},
    {role: occupancy_image, name, size_bytes, sha256}
  ]
}
map_content_hash = SHA256(canonical_json(map_hash_manifest))

# 49. Lifecycle startup order

Mapping: robot description -> drivers/bridge -> EKF -> SafetySupervisor -> slam_toolbox -> enable teleop.

Navigation: description -> drivers/bridge -> EKF -> Map Server -> AMCL -> LocalizationMonitor valid -> Planner/global costmap -> Safety -> selected controller/manager -> accept mission.

A downstream motion source remains inhibited until every upstream lifecycle and contract check is healthy.

# 50. Full verification obligations

TF/estimation: single map->odom, single odom->base_link theo profile integration/deployment, no world->base_link GT TF, wheel order/sign/counter wrap/packet loss/MCU reboot tests; sim_train MVP không tạo TF authority mới và policy không dùng GT measured twist.

Mapping/localization: teleop through safety, approved map/hash, one provider, initial pose, covariance, pose jump and map identity tests.

Navigation: planner lifecycle, empty/wrong/stale path, cancel/preemption/replan, lookahead intersection, LocalReference timestamp and position-only settle tests.

Command/safety: only one /cmd_vel publisher, mode/source/epoch/generation/instance/sequence/TTL/finite checks, holonomic vx/vy/wz swept footprint, watchdog and physical E-stop independence.

Evaluation: same approved map, goals, scenarios, seeds and hard safety envelope for Nav2 baseline and hybrid; sealed splits; ground truth cannot influence policy. sim_train phải chứng minh ground-truth isolation, episode-seed reset của bias/drift/delay queue và reproducibility của sampled-parameter manifest.

Navigation-state tests: missing/stale state, process restart, backward/duplicate sequence, wrong epoch/generation, invalid/expired path và motion_permitted=false đều tạo zero command; REPLANNING chỉ được tiếp tục với path cũ còn hợp lệ.

# 51. Definition of Done và lộ trình triển khai

Không còn frame contradiction; mọi profile có pose source, goal frame và TF authority duy nhất. sim_train MVP dùng internal SimulatedOdometryEmulator snapshot và chưa thêm TF authority mới.

Mọi required topic có exact type, QoS, owner, clock/frame, profile và stale behavior.

Bốn overlay resolve thành immutable config/hash; runtime/model/checkpoint/report cùng hash.

Safety snapshot đầy đủ; contact state machine và reset epoch test được; /cmd_vel chỉ có một publisher.

Package skeleton chỉ được build-ready sau khi REQUIRED_BUILD_INPUT resolve; validators trả errors cụ thể.

Test/traceability phủ requirement; placeholder audit chỉ còn ba taxonomy hợp lệ.

Trạng thái cuối của tài liệu: ARCHITECTURE_FROZEN, IMPLEMENTATION_READY, NOT_YET_REAL_ROBOT_VALIDATED.

Phase 0: freeze interfaces/config schemas, Pi_STM32_Interface_Contract and package ownership.

Phase 1: update interfaces; implement mecanum_base_bridge telemetry/wheel odometry and tests.

Phase 2: implement mecanum_nav_bringup, EKF, TF/QoS/startup validators and localization monitor.

Phase 3: implement simulator adapter gồm SimulatedOdometryEmulator nội bộ, transactional reset, ground-truth isolation tests, core env/observation/action/reward/termination và Gate 1.

Phase 4: mapping/map registry, AMCL Omni, Nav2 baseline and hybrid path/reference manager.

Phase 5: PPO training/checkpoint/evaluator and fair Nav2-vs-hybrid Gate 2 evaluation.

Phase 6: domain randomization, full hybrid simulation, map/command fault injection and Gate 3 evidence.

Phase 7: HIL, measurements, staged real tests and Gate 4 before deploy_real.

# Phụ lục A. Cấu hình base và bốn overlays

# mecanum_nav_rl/config/base.yaml
schema_version: "3.0"
runtime: {profile: REQUIRED_RUNTIME_PROFILE, navigation_mode: REQUIRED_NAVIGATION_MODE}
robot: {kinematics: holonomic, base_frame: base_link}
observation:
  schema_id: obs-v3-l72-g3-t3-c3-f32
  shape: [81]
  dtype: float32
  lidar: {sectors: 72, reduction: angular_sector_min}
  goal_features: [normalized_distance, sin_bearing, cos_bearing]
action:
  schema_id: action-v3-holonomic-f32
  shape: [3]
  normalized_bounds: [-1.0, 1.0]
navigation:
  require_goal_orientation: false
  goal_distance_scale_m: REQUIRED_BUILD_INPUT
  lookahead_distance_m: REQUIRED_BUILD_INPUT
  max_reference_snapshot_skew_s: REQUIRED_BUILD_INPUT
  goal_position_tolerance_m: TBD_PROJECT_ACCEPTANCE
  goal_stopped_linear_speed_mps: TBD_PROJECT_ACCEPTANCE
  goal_stopped_angular_speed_radps: TBD_PROJECT_ACCEPTANCE
  goal_settle_time_s: TBD_PROJECT_ACCEPTANCE
command:
  message_type: mecanum_nav_rl_interfaces/msg/CommandEnvelope
  ttl_real_s: TBD_MEASURED
map:
  source_root: artifacts/maps
  hash_schema: mecanum_map_hash/v1

# config/profiles/sim_train.yaml
runtime: {profile: sim_train, navigation_mode: local_training, use_sim_time: true}
policy_pose_source: noisy_simulated_odometry
ground_truth_policy_access: false
ground_truth_task_oracle_access: true
simulated_odometry: {output: internal_typed_snapshot, public_topic: none, tf_publisher: none, reset: episode_seed, parameter_manifest: required}
lidar: {model: RPLIDAR_A1M8_SIM, frame: RPLiDAR_A1M8_1, sectors: 72}

# deploy_real overlay
runtime: {profile: deploy_real, use_sim_time: false}
lidar: {model: YDLIDAR_X3, frame: REQUIRED_BUILD_INPUT}
state_estimation: {owner: robot_localization_ekf, two_d_mode: true}
localization: {provider: amcl, motion_model: nav2_amcl::OmniMotionModel}
map: {map_id: REQUIRED_BUILD_INPUT, expected_hash: REQUIRED_BUILD_INPUT}
real_motion_limits: TBD_MEASURED
command_ttl_s: TBD_MEASURED
watchdog_timeout_s: TBD_MEASURED
stopping_envelope: TBD_MEASURED
gate4_required: true

# Phụ lục B. Custom message definitions

Bốn custom messages được chia sẻ qua DDS: Heartbeat, SafetyState, NavigationState và CommandEnvelope. Rich internal snapshots remain immutable Python dataclasses.

# Heartbeat.msg
uint8 SOURCE_UNKNOWN=0
uint8 SOURCE_POLICY=1
uint8 SOURCE_MCU=2
uint32 FAULT_NONE=0
uint32 FAULT_TRANSPORT=1
uint32 FAULT_ESTOP_ACTIVE=2
uint32 FAULT_POLICY_INFERENCE=4
uint32 FAULT_POLICY_COMMAND_INVALID=8
uint32 FAULT_POLICY_CONTROL_LOOP=16
uint32 FAULT_POLICY_MODEL_CONTRACT=32
uint32 FAULT_MCU_COMMAND_TIMEOUT=256
uint32 FAULT_MCU_COMMAND_INVALID=512
uint32 FAULT_MCU_CONTROL_LOOP=1024
uint32 FAULT_MOTOR_DRIVER=2048
uint32 FAULT_ENCODER=4096
uint32 FAULT_IMU=8192
std_msgs/Header header
uint8 source
uint32 sequence
uint32 reset_epoch
bool healthy
uint32 fault_bits
string detail

# SafetyState.msg
uint8 STATE_INIT=0
uint8 STATE_NORMAL=1
uint8 STATE_LIMITED=2
uint8 STATE_STOPPED=3
uint8 STATE_ESTOPPED=4
uint8 STATE_FAULT=5
uint8 COMMAND_SOURCE_NONE=0
uint8 COMMAND_SOURCE_POLICY=1
uint8 COMMAND_SOURCE_NAV2=2
uint8 COMMAND_SOURCE_TELEOP=3
uint8 COMMAND_SOURCE_SAFE_STOP=4
uint8 COMMAND_SOURCE_ESTOP=5
std_msgs/Header header
uint8 state
uint32 reset_epoch
uint32 runtime_generation
uint8 active_command_source
bool command_valid
bool scan_fresh
bool odom_fresh
bool tf_valid
bool localization_valid
bool navigation_state_fresh
bool path_valid
bool motion_permitted
bool policy_alive
bool mcu_alive
bool estop_active
uint32 scan_age_ms
uint32 odom_age_ms
uint32 policy_heartbeat_age_ms
uint32 mcu_heartbeat_age_ms
uint32 command_age_ms
uint32 navigation_state_age_ms
uint32 fault_bits
string[] intervention_reasons
geometry_msgs/Twist raw_command
geometry_msgs/Twist limited_command
geometry_msgs/Twist final_command

# NavigationState.msg
uint8 STATE_IDLE=0
uint8 STATE_PLANNING=1
uint8 STATE_FOLLOWING=2
uint8 STATE_REPLANNING=3
uint8 STATE_STOPPING=4
uint8 STATE_SUCCEEDED=5
uint8 STATE_CANCELED=6
uint8 STATE_FAILED=7
uint32 ERROR_NONE=0
uint32 ERROR_INVALID_GOAL=1
uint32 ERROR_LOCALIZATION_INVALID=2
uint32 ERROR_PLANNER_UNAVAILABLE=3
uint32 ERROR_PLANNING_FAILED=4
uint32 ERROR_PATH_INVALID=5
uint32 ERROR_PATH_EXPIRED=6
uint32 ERROR_TF_FAILURE=7
uint32 ERROR_INTERNAL=8
std_msgs/Header header
uint8 state
string goal_id
uint64 path_sequence
uint32 reset_epoch
uint32 runtime_generation
uint64 source_instance_id
uint64 sequence
bool path_valid
bool motion_permitted
float32 distance_remaining_m
uint32 error_code
string detail

# Contract: header.frame_id=map. distance_remaining_m is finite and >=0 when path_valid;
# otherwise it is exactly -1.0. Machine logic never parses detail.
# Safety computes freshness from local steady receive time; it does not trust ROS time for node loss.
# motion_permitted is necessary but never sufficient to move.

# CommandEnvelope.msg
uint8 SOURCE_UNKNOWN=0
uint8 SOURCE_POLICY=1
uint8 SOURCE_NAV2=2
uint8 SOURCE_TELEOP=3
uint8 SOURCE_SAFE_STOP=4
std_msgs/Header header
uint8 source
uint64 sequence
uint32 reset_epoch
uint32 runtime_generation
uint64 source_instance_id
geometry_msgs/Twist twist

Header stamp uses the profile ROS clock. Safety records received_steady_ns locally; absolute steady time is never transmitted as if clocks were shared across hosts.

SafetyState header.frame_id=base_link. Heartbeat header.frame_id để rỗng; freshness dựa received steady time, không chỉ header stamp.

Thay field/constants là interface version change; rosidl package version và protocol hash phải tăng.

# Phụ lục C. Package skeleton

# mecanum_nav_rl_interfaces v2.4.0
ament_cmake + rosidl_default_generators; std_msgs and geometry_msgs dependencies.
rosidl_generate_interfaces: Heartbeat.msg, SafetyState.msg, NavigationState.msg, CommandEnvelope.msg.
package.xml version: 2.4.0; maintainer: Ha Van Vu <havan.vu1302@gmail.com>; license: Apache-2.0.
CHANGELOG.md records the public contract change. test_interface_contracts.py verifies constants, fields, imports and NavigationState safety semantics.

# Official ROS packages
ROBOT_URDF_final_description: robot/simulation description.
mecanum_nav_rl_interfaces: typed cross-process interfaces.
mecanum_base_bridge: Pi-STM32 transport and wheel odometry without TF.
mecanum_nav_rl: DRL, hybrid navigation, safety, training and evaluation.
mecanum_nav_bringup: EKF, SLAM, AMCL, Nav2, safety configs and system launch.

mecanum_nav_rl adds nav2_msgs and diagnostic_msgs runtime dependencies.
mecanum_nav_bringup depends on robot_localization, slam_toolbox and Nav2 packages.
Each package keeps its own package.xml/build file and installs only owned assets.

mecanum_nav_rl setup.py installs only owned Python/config/scenario assets. System launch and external-stack YAML are installed by mecanum_nav_bringup; maps remain artifact paths supplied at launch.

Project metadata đã được resolve thành Ha Van Vu, havan.vu1302@gmail.com và Apache-2.0. Các package mới vẫn phải vượt colcon build và CI trước khi được gọi build-tested.

# Phụ lục D. Blocking inputs và checklist triển khai

## D.1 Danh sách TBD_MEASURED

Real velocity/acceleration/jerk envelope theo forward, reverse, lateral, diagonal và yaw.

Sensor-to-actuator latency p95/p99/worst, command jitter và sensor age bound.

Braking distance/deceleration theo speed, direction, payload, surface và battery.

LiDAR noise, dropout, range bias, invalid-ray distribution và effective update timing.

Localization/odom uncertainty, loss modes, TF alignment error và wheel slip/traction.

Command TTL, watchdog timeout, source heartbeat timeout, stop/slow TTC và engineering reserve.

Physical footprint/collision geometry và E-stop/MCU restart end-to-end behavior.

## D.2 Danh sách TBD_PROJECT_ACCEPTANCE

Evaluation seeds; episodes per scenario; minimum success rate.

Maximum collision, timeout, stuck, infrastructure-failure và safety-intervention rates.

Minimum path efficiency; maximum time-to-goal và control jerk RMS.

Confidence level/interval procedure và baseline comparison rule.

Mild/measured robustness retention hoặc absolute thresholds nếu được dùng.

Owner: Principal Investigator; deadline: trước khi unseal final_test; artifact phải có approval và hash.

## D.3 Danh sách REQUIRED_BUILD_INPUT

Resolved project metadata: Ha Van Vu, havan.vu1302@gmail.com, Apache-2.0.

Absolute/ament-share robot Xacro path, world/model/entity identifiers và expanded description checksum.

Monitored body links, exact fully scoped collision names và generated contact contract.

Verified footprint source/vertices và static base_link->lidar_frame geometry cho cả RPLiDAR simulation và YDLIDAR X3 thật.

Exact Gazebo Harmonic contact plugin identifier available in installed distribution và bridge launch config.

Localization/odom node authority names cho sim_eval/deploy_sim/deploy_real; Nav2 `enable_stamped_cmd_vel=false` resolved parameter evidence.

Artifact root, scenario manifest root, hardware/firmware IDs và MCU bridge transport selection (micro-ROS hoặc serial bridge).

## D.4 Ba lượt audit trước merge

| Audit | Checklist pass |
| --- | --- |
| Consistency | Topic-type-QoS-owner; frame-pose-profile; 81-D obs; reward-context; Gym flags; checkpoint hashes; module-file owner |
| Sim-to-Real | Policy deploy không có GT; same encoder/action/safety path; measured randomization provenance; no sim limit reused as real |
| Placeholder | Chỉ TBD_MEASURED, TBD_PROJECT_ACCEPTANCE, REQUIRED_BUILD_INPUT; target-specific compiler rejects unresolved tokens |

## D.5 Implementation kickoff checklist

[ ] Bootstrap/cập nhật đủ năm ROS packages và chạy colcon/CI; project metadata đã resolve.

[ ] Chạy collision extractor; commit generated contract và checksums.

[ ] Implement ConfigCompiler/validators trước Gym environment.

[ ] Implement clocks/buffers/contact/reset và pass Gate 1 contract tests.

[ ] Refactor prototype vào pure observation/action/reward/termination modules.

[ ] Implement training/eval/recovery; preregister acceptance trước final evaluation.

[ ] Implement safety process/HIL; thu M01-M12; không bật deploy_real trước Gate 4.

# Tài liệu chính thức dùng để khóa quyết định

ROS 2 Jazzy - Executors

ROS 2 Jazzy - QoS

ROS 2 Jazzy - diagnostic_msgs

Nav2 Jazzy - Velocity Smoother and enable_stamped_cmd_vel

Nav2 Jazzy - Collision Monitor

Gymnasium Env API

Stable-Baselines3 Callbacks

Gazebo Harmonic ROS 2 integration

ros_gz_bridge supported message mappings

ROS REP-105 - Coordinate Frames for Mobile Platforms

Nav2 Jazzy - robot_localization odometry setup

Nav2 Jazzy - AMCL OmniMotionModel

Nav2 Jazzy - ComputePathToPose and FollowPath actions

SLAM Toolbox - mapping, serialization and localization

# Kết luận và điều kiện triển khai

| Mục | Kết luận |
| --- | --- |
| Architecture | ARCHITECTURE_FROZEN |
| Implementation | IMPLEMENTATION_READY |
| Real robot | NOT_YET_REAL_ROBOT_VALIDATED |
| Được phép | Implement packages/modules/tests theo contract; train/evaluate trong simulation sau Gate 1 |
| Chưa được tuyên bố | Gate 3/4 pass; Sim-to-Real success; real operational safety; functional safety certification |
| Khóa deploy_real | All TBD_MEASURED VALID, approved map/model, HIL and Gate 4 pass |

Quyết định: Đây là bản kiến trúc chính thức dùng trước giai đoạn lập trình. Mọi thay đổi làm biến đổi public protocol, topic/QoS/TF contract, observation/action/reward schema hoặc safety boundary phải tạo Architecture Change Record và tăng phiên bản phù hợp.

Bắt đầu Phase 0 bằng việc cập nhật interfaces/config schemas và khóa Pi_STM32_Interface_Contract; tiếp tục core hiện có, không viết lại các module đã pass test.

Không mở sealed final_test trước khi AcceptanceCriteriaConfig được phê duyệt.

Không bật deploy_real trước khi map/model được phê duyệt, measurement registry VALID, HIL pass và Gate 4 đạt; tài liệu không tuyên bố functional safety certification.